Hydration Monitoring System

Abstract

The Arduino-based Hydration Monitoring System presented here utilizes graphite electrodes as skin conductivity sensors along with a 16x2 I²C LCD display to provide a low-cost, non-invasive technique for monitoring hydration levels. By measuring changes in skin conductance—correlated with the body's hydration status—the Arduino platform processes sensor data, applies filtering, and outputs real-time hydration percentages. The results showcase technical feasibility, affordable parts, and potential real-world applications in healthcare and sports science.​

Introduction

Sustained hydration is essential for human health, impacting cognitive performance, skin condition, organ function, and physical endurance. Traditional hydration monitoring relies on subjective methods (e.g., thirst perception or urine color) and expensive lab-grade sensors (e.g., bioimpedance analyzers), limiting their accessibility for personal or field use. Wearable hydration sensors, designed to be non-invasive and low-cost, are an emerging solution, utilizing skin conductivity as a measurable proxy for hydration. By building such a system with Arduino, this project addresses the need for affordable, accessible, and user-friendly hydration monitoring to support personal health, sports medicine, and remote patient care.​

Objective

The core aim of this project is to design, implement, and validate an Arduino-based hydration monitoring system using graphite electrodes to measure skin conductivity and a 16x2 I²C LCD for real-time hydration feedback. The system targets affordability, simplicity, reasonable accuracy, and suitability for personal or small-scale health monitoring applications.​

Components Used

- Arduino Uno: Main controller for data acquisition, processing, and display control.​

- 16x2 I²C LCD Display: Provides real-time, readable hydration output; uses only two wires for simplified connection.​

- Graphite Leads: Used as skin-contact electrodes to sense electrical conductance changes.

- 4.7k Potentiometer: Adjusts the voltage divider, allowing sensitivity tuning of the conductivity circuit.

- Jumper Wires: For flexible circuit connections.

- Breadboard: Rapid prototyping and component installation.

- Optional - Aluminum Foil: Can substitute electrodes for capacitive or larger contact area for experimental purposes.​

- Miscellaneous: Resistors (e.g., 10kΩ, 220Ω) for extra noise reduction or circuit testing.

Working Principle

The system operates by applying a small voltage across two graphite electrodes contacting skin. Conductance, influenced by skin moisture (hydration), changes as water content increases or decreases; hydrated skin conducts more electrical current due to higher ionic content. The Arduino reads voltage drop across a potentiometer–graphite divider circuit, converts analog signals to digital (ADC), and mathematically maps the conductance (inverse of resistance) into a hydration percentage. To improve accuracy, the data undergoes moving-average filtering before display.​

Circuit Design & Connections

- Electrode Divider: The graphite leads connect between 5V and a potentiometer, then to analog pin A0. The potentiometer changes the sensitivity and operating range.

- Breadboard Layout: Components laid out for easy access and interchangeability during testing.

- LCD Wiring: The I²C LCD module connects SDA and SCL pins to Arduino A4 and A5 (on Uno), sharing power (5V, GND).​

- Optional: Aluminum foil electrodes or capacitive setups may be experimented with for broader contact and alternate sensing.

- Pin Assignments Example:

  - 5V: Electrode 1 (reference)

  - A0: Electrode 2 (signal)

  - SDA (LCD): A4

  - SCL (LCD): A5

  - GND: Shared ground for all components

Software Implementation

- Analog Reading: Arduino’s analogRead(A0) reads the voltage from the sensor divider.

- Mapping to Hydration Percentage: Raw readings are normalized and mapped using the map() function to convert sensor range to 0–100%.​

- Moving-Average Filter: Smoothens noisy data using a windowed average (e.g., 5-sample buffer).​​

- LCD Display Update: Processed readings are converted to hydration percentages and sent to the 16x2 LCD using the LiquidCrystal_I2C library.​

- Sample Code Logic:

text

int sensorPin = A0;

int readings[5];  // Moving average buffer

int sum = 0; int index = 0;

int hydrationPercent = 0;

void loop() {

int rawValue = analogRead(sensorPin);

sum = sum - readings[index];

readings[index] = rawValue;

sum = sum + rawValue;

index = (index + 1) % 5;

int avgValue = sum / 5;

hydrationPercent = map(avgValue, minDry, maxWet, 0, 100);

lcd.setCursor(0,0);

lcd.print("Hydration: ");

lcd.print(hydrationPercent);

lcd.print("%");

delay(500);

}

  - Parameters such as window size and mapping range require calibration for accurate results.​

Testing & Observations

- Testing Steps:

  - System powered on, electrodes placed on subject’s skin (preferably fingers or forearm).

  - Measurements taken under two conditions: dry skin (unwashed, low moisture) and hydrated skin (post hand-wash, topical water application).

  - Potentiometer adjusted to optimize sensitivity and baseline offset.

- Sample Readings:

  - Dry skin: Lower ADC readings, mapped to 20–40% hydration.

  - Hydrated skin: Noticeable jump in readings, mapped to 70–100% hydration.

- Observations: Sensitivity can be fine-tuned by turning the potentiometer; moving average filter makes readings steadier.​

Results & Discussion

- Results: The system successfully displayed higher hydration percentages for wet or moisturized skin and lower for dry conditions. Values tracked relative hydration rather than absolute water volume.

- Accuracy: Dependent on electrode quality, skin type, and environmental factors; within ±10% for basic differentiation.​

- Limitations:

  - Calibration needed for individual skin types and ambient temperature.

  - Graphite electrodes can oxidize/foul over time.

  - Simple voltage divider lacks true current regulation.

  - LCD display may lag if filtering window is too broad.

- Possible Improvements: Adding multi-point calibration, better electrodes, or analog front-end ICs could increase accuracy. Consideration for environmental noise and user variability is important.​

Applications

- Sports Science: Tracks hydration in athletes during competition/training.​

- Medical Monitoring: Early dehydration warning for elderly and clinical patients.

- Personal Health Devices: Daily self-monitoring; integration into wearable patches or wristbands.

- Other Potential Uses: Research on sweat rate, stress monitoring, or environmental adaptation.​

Future Scope

- Bluetooth/Wireless Integration: Send results to a smartphone app for logging and notification.

- Automatic Calibration: Self-adjustment for baseline skin resistance.

- Capacitive Sensing: Combine resistive and capacitive measurement using foil electrodes for dual-mode hydration/moisture analysis.​

- Wearable Design: Miniaturize and integrate into flexible substrate or e-textile for continuous wear.

- Advanced Filtering: Real-time adaptive signal processing for noisy environments.​

Conclusion

The Arduino-based Hydration Monitoring System demonstrates a practical, low-cost approach to personal hydration assessment using widely available graphite electrodes and an I²C LCD output. While basic, it facilitates real-time monitoring, encourages healthy habits, and lays groundwork for future wearable tech. Continued refinement and integration with wireless, calibration, and multi-modal sensing can enable even broader applications for non-invasive hydration monitoring.​

References

- Arduino Forum. "Skin Conductivity Circuit – HELP PLS!"​

- Kosch, T. "Galvanic Skin Response powered by Arduino."​

- Maker Pro. "How to Clean Up Noisy Sensor Data With a Moving Average Filter."​

- Instructables. "Arduino Temp/Humidity Sensor Using DHT11 and I2C LCD One Day Project."​

- Wu, H. et al. "Materials, Devices, and Systems of On‐Skin Electrodes for ..." Advanced Science, 2021.​

- Hackster.io. "Smart Glass Counter - Arduino Hydration Tracker Project."​